COMMON COURSE ASSIGNMENT

Department of Computer Science and Engineering

# A. Assignment Information

| Department | Computer Science and Engineering |
| --- | --- |
| Programme | B.Tech |
| Course Code & Course Name | CSA0702 – Computer Networks |
| Academic Year / Batch | 2025 |
| Faculty Name | Dr. Senthamilselvan R |
| Assignment Title | Design a VLAN-based enterprise LAN containing separate VLANs for Administration, Faculty, Students, and Guests. Configure VLANs and verify communication. |
| Date of Issue | 02/09/2026 |
| Date of Submission | 02/09/2026 |
| Maximum Marks | 100 |
| Course Outcome(s) – CO | CO1: Identify and explain fundamental networking concepts, network devices, topologies, protocols, and layered network architectures. CO2: Configure IP addressing, subnetting, switching, and routing techniques for establishing computer networks using Cisco Packet Tracer. CO3: Analyze network traffic, addressing information, VLAN communication, and connectivity using appropriate network simulation and packet-analysis tools. CO4: Evaluate network performance, connectivity, segmentation, and security requirements for different enterprise networking scenarios. CO5: Design and implement VLAN-based enterprise networks by creating separate VLANs for Administration, Faculty, Students, and Guests, configuring inter-VLAN communication, and verifying connectivity using Cisco Packet Tracer. |
| Bloom's Taxonomy Level | L6 – Design; L5 – Evaluate; L4 – Analyze; L3 – Configure; L2 – Explain |
| SDG Mapping (SDG 1–17) | SDG 4 – Quality Education: Develops practical networking knowledge, digital skills, analytical abilities, and hands-on technical competency. SDG 9 – Industry, Innovation and Infrastructure: Promotes the design and implementation of reliable, secure, scalable, and innovative ICT/network infrastructure. SDG 11 – Sustainable Cities and Communities: Provides networking skills relevant to connected campuses, smart buildings, smart cities, and resilient digital services. |
| Industry / Societal Relevance | Enterprise LAN segmentation using VLANs is a standard industry practice for improving network security, traffic management, and scalability in organizational and campus network deployments. |

# B. Assignment Problem / Challenge

# Apply course concepts rather than reproduce theory: Students must actively configure VLANs, subnetting, trunking, and inter-VLAN routing in Packet Tracer rather than simply define these terms — the deliverable is a working, verified network, not a written explanation of VLAN theory.

# Identify and analyze the problem: Students must recognize that a flat, unsegmented LAN across four departments creates security and broadcast-domain issues, and translate that into a segmentation requirement before any configuration begins.

# Consider requirements and constraints: The assignment requires an explicit IP addressing scheme, correct VLAN-to-port assignment, and restricted access for Guests — all of which force students to work within functional, security, and addressing constraints rather than an open-ended design.

# Develop or compare alternative solutions where appropriate: Students are required to compare Router-on-a-Stick vs. Layer 3 switch for inter-VLAN routing, and justify which is more appropriate for this scenario.

# Use relevant modern engineering/computing/design tools: Cisco Packet Tracer is used throughout for design, configuration, and simulation — directly satisfying CO2, CO3, and CO5.

# Interpret results: Students must read and interpret show vlan brief, show interfaces trunk, and ping/traceroute outputs to confirm the design is functioning as intended, not just run commands blindly.

# Make and justify engineering decisions: The choice of VLSM over fixed subnetting, Router-on-a-Stick over a Layer 3 switch, and ACL-based restriction over full isolation all require a justified engineering decision rather than an arbitrary choice.

# Consider technical, economic, environmental, societal, ethical, safety or sustainability factors: The assignment ties directly into SDG 4, 9, and 11 — addressing sustainability/scalability of the address scheme, security/access-control responsibility, and the broader societal role of resilient campus network infrastructure.

# C. Problem Statement

Problem / Case / Design Challenge:

Design and implement a VLAN-based enterprise Local Area Network (LAN) using Cisco Packet Tracer for an organization with four departments: Administration, Faculty, Students, and Guests. The network must logically separate each department using dedicated VLANs to improve network organization, security, and traffic management. Configure appropriate VLANs, assign switch ports to the respective VLANs, configure IP addressing and inter-VLAN communication, and verify connectivity between authorized network segments. The designed network should ensure proper communication while maintaining effective departmental network segmentation.

This is the same Problem Statement text already used in Section C of the document (from the original generation) — no changes needed there, it's already correctly in place.

At this point we've now walked through every part of the assignment:

- A – Assignment Information

- B – Assignment Problem/Challenge (with the criteria mapping you just confirmed)

- C – Problem Statement (this one)

- D – Requirements and Constraints

- E.1–E.10 – full Student Work section (Problem Understanding → References)

- F – Assessment Rubric

- G – CO–PO Mapping

- H – Faculty Evaluation template

# D. Requirements and Constraints

Students shall identify or be provided with the relevant requirements and constraints applicable to this design problem, including:

- Functional requirements — separate VLANs for Administration, Faculty, Students, and Guests; inter-VLAN routing where authorized.

- Performance requirements — reliable connectivity and minimal latency between authorized segments.

- Technical constraints — use of Cisco Packet Tracer for simulation, appropriate switch and router configuration.

- Security/privacy — restriction of unauthorized inter-departmental access (e.g., Guests isolated from Administration).

- Resources — number of switches, routers, and end devices available for the simulation.

- Standards/codes — standard VLAN tagging (IEEE 802.1Q) and IP addressing conventions.

- Scalability/sustainability — network design should accommodate future expansion of departments or users.

Only constraints relevant to this problem need to be considered.

# E. Student Work

1. Problem Understanding and Formulation

What is the problem?

The organization currently runs (or is planned to run) a single flat LAN in which all devices — belonging to Administration, Faculty, Students, and Guests — share the same broadcast domain. This causes unnecessary broadcast traffic, weak access control between departments, and no way to isolate sensitive traffic (e.g., Administration) from less trusted traffic (e.g., Guests). The problem is to logically segment this single network into four separate VLANs, configure the switches and IP addressing for each, enable inter-VLAN routing only where authorized, and verify that the resulting network communicates correctly while keeping departments properly isolated.

What are the expected outcomes?

- Four functioning VLANs (Administration, Faculty, Students, Guests), each in its own broadcast domain.

- Each switch port correctly assigned to its department's VLAN.

- A working IP addressing scheme with one subnet per VLAN.

- Inter-VLAN routing configured (via Router-on-a-Stick or a Layer 3 switch) so that only authorized communication between VLANs is possible.

- Verified connectivity: devices within a VLAN can reach each other; devices across VLANs can reach each other only where permitted (e.g., Guests blocked from Administration).

- Evidence of successful configuration and testing captured in Cisco Packet Tracer (topology, configs, ping/traceroute results).

What information/data is available?

- The four department names requiring separate VLANs: Administration, Faculty, Students, Guests.

- The requirement that VLANs must be configured and inter-VLAN communication verified.

- Implicit standard data: number of end devices per department (to be assumed/decided), switch and router models available in Packet Tracer, and the general enterprise LAN context (single site, single building/campus).

- No specific IP ranges, VLAN ID numbers, or device counts are given in the problem statement — these must be assumed by the student

What assumptions are made?

- VLAN IDs are assigned by the student, e.g., VLAN 10 = Administration, VLAN 20 = Faculty, VLAN 30 = Students, VLAN 40 = Guests.

- A private IP addressing scheme is used, e.g., 192.168.10.0/24, 192.168.20.0/24, 192.168.30.0/24, 192.168.40.0/24 for the four VLANs respectively.

- A small representative number of end devices (e.g., 2–3 PCs) is used per VLAN for simulation purposes, rather than a full-scale enterprise deployment.

- Inter-VLAN routing is implemented using either a Router-on-a-Stick configuration (sub-interfaces with 802.1Q trunking) or a Layer 3 switch, since Packet Tracer supports both.

- The Guests VLAN is assumed to require restricted access (e.g., internet-only or no access to Administration/Faculty), reflecting standard enterprise security practice, even though this isn't explicitly stated in the problem.

- A single access switch (or a small switch stack) connects to a central router/Layer 3 switch, forming a typical single-building enterprise topology.

What constraints must be satisfied?

- All four VLANs must be correctly created and named on the switch(es).

- Switch ports must be correctly assigned to their respective VLANs (access ports) and trunk ports configured where needed.

- IP addressing must not overlap between VLANs and must allow correct routing.

- Inter-VLAN communication must work only for authorized paths — unauthorized cross-department access (e.g., Guests to Administration) should be prevented or restricted.

- The design and configuration must be achievable and verifiable within Cisco Packet Tracer.

- Standard VLAN tagging (IEEE 802.1Q) conventions must be followed for trunk links.

2. Application of Course Knowledge

Principles

- Network segmentation principle — dividing a single broadcast domain into multiple logical domains (VLANs) reduces broadcast traffic, improves security, and isolates faults to a single segment.

- Layered network architecture (OSI/TCP-IP model) — VLANs operate at Layer 2 (data link layer, switching/frame tagging), while inter-VLAN communication requires Layer 3 (network layer, routing) intervention.

- Principle of least privilege — the Guests VLAN is restricted from reaching sensitive VLANs (Administration/Faculty), applying access-control at the network level.

Theories

- IEEE 802.1Q VLAN tagging standard — theoretical basis for how frames are tagged with a VLAN ID as they cross trunk links between switches, allowing a single physical link to carry traffic for multiple VLANs.

- CIDR (Classless Inter-Domain Routing) theory — replaces classful addressing with variable-length prefixes, enabling efficient allocation of address space to each VLAN based on actual host requirements.

- Spanning Tree Protocol (STP) theory — prevents Layer 2 loops in switched networks with redundant links, relevant if the enterprise topology includes multiple switches.

Mathematical Models

- Subnetting formula: number of usable hosts per subnet = 2ⁿ − 2, where n = number of host bits.

- VLSM (Variable Length Subnet Masking) allocation model: subnets are sized to match actual host-count requirements rather than a fixed uniform size, minimizing address wastage.

- CIDR notation model: subnet mask expressed as /prefix, where prefix = number of network bits (32 − n).

Algorithms

- VLSM allocation algorithm — sort departments by host requirement in descending order, then allocate the smallest sufficient block to each in turn, so the largest subnet is carved first from the available address space.

- Longest Prefix Match (LPM) algorithm — used by the router/Layer 3 switch to determine which VLAN subnet a destination IP belongs to when forwarding inter-VLAN traffic.

- Spanning Tree Algorithm (STA) — computes a loop-free logical topology when redundant switch links exist.

Engineering Concepts

- Router-on-a-Stick / Layer 3 switching — a single router interface (using sub-interfaces) or a Layer 3 switch handles routing between multiple VLANs over a trunk link.

- Trunking vs. access ports — access ports carry traffic for a single VLAN (end devices); trunk ports carry tagged traffic for multiple VLANs (switch-to-switch, switch-to-router links).

- Access Control Lists (ACLs) — used to enforce restricted inter-VLAN communication (e.g., blocking Guests from reaching Administration).

Scientific Concepts

- Binary number representation — IP addresses and subnet masks are fundamentally binary values; subnetting calculations depend on binary AND operations between address and mask.

- Collision/broadcast domain theory — switching confines collision domains to each port, while VLANs confine broadcast domains to each logical segment.

Design Methodologies

- Top-down network design approach — start from organizational/business requirements (four departments needing isolation) before deciding technical details (VLAN IDs, IP scheme, device configuration).

- Structured VLAN and IP addressing plan — assign a consistent, documented scheme (VLAN ID ↔ department ↔ subnet) before configuration, rather than ad hoc assignment.

Calculations: VLAN and IP Addressing Scheme (VLSM)

Assume the organization has been allocated the address block 192.168.1.0/24 (256 addresses) and the following estimated host requirements:

| Department | VLAN ID | Estimated Hosts |
| --- | --- | --- |
| Students | 30 | 50 |
| Faculty | 20 | 30 |
| Administration | 10 | 20 |
| Guests | 40 | 10 |

Step 1 — Sort by host requirement (largest first) for VLSM:
Students (50) → Faculty (30) → Administration (20) → Guests (10)

Step 2 — Determine subnet size using 2ⁿ − 2 ≥ required hosts:

- Students (50 hosts): need 2ⁿ − 2 ≥ 50 → n = 6 (2⁶ − 2 = 62 usable) → /26 (255.255.255.192)

- Faculty (30 hosts): need 2ⁿ − 2 ≥ 30 → n = 5 (2⁵ − 2 = 30 usable) → /27 (255.255.255.224)

- Administration (20 hosts): need 2ⁿ − 2 ≥ 20 → n = 5 (2⁵ − 2 = 30 usable) → /27 (255.255.255.224)

- Guests (10 hosts): need 2ⁿ − 2 ≥ 10 → n = 4 (2⁴ − 2 = 14 usable) → /28 (255.255.255.240)

Step 3 — Allocate blocks sequentially from 192.168.1.0/24:

| VLAN | Department | Subnet | Mask | Usable Range | Broadcast |
| --- | --- | --- | --- | --- | --- |
| 30 | Students | 192.168.1.0/26 | 255.255.255.192 | .1 – .62 | .63 |
| 20 | Faculty | 192.168.1.64/27 | 255.255.255.224 | .65 – .94 | .95 |
| 10 | Administration | 192.168.1.96/27 | 255.255.255.224 | .97 – .126 | .127 |
| 40 | Guests | 192.168.1.128/28 | 255.255.255.240 | .129 – .142 | .143 |

Remaining space, 192.168.1.144 – .255, is reserved for future departments or network growth — demonstrating the efficiency of VLSM over a fixed uniform /26 or /27 scheme for all four VLANs.

Step 4 — Gateway assignment: the first usable address in each subnet is reserved as the default gateway (sub-interface on the router, or SVI on the Layer 3 switch), e.g., 192.168.1.1 for Students, 192.168.1.65 for Faculty, and so on.

3. Solution / Design / Methodology

The diagram above shows the overall topology: a router (or Layer 3 switch) sits above a core switch, connected by a trunk carrying all four VLANs, which fan out to the four department segments. Here's the full solution design to go with it.

Network Topology Diagram

(shown above) — Router/L3 switch → Core switch (802.1Q trunk) → four access-layer VLANs (Admin, Faculty, Students, Guests). Each end-device connects to an access switch port assigned to its department's VLAN; the trunk link between the core switch and the router/L3 switch carries tagged traffic for all four VLANs simultaneously.

VLAN Configuration Plan

| VLAN ID | Name | Department | Port Type |
| --- | --- | --- | --- |
| 10 | ADMIN | Administration | Access ports on assigned switch ports |
| 20 | FACULTY | Faculty | Access ports on assigned switch ports |
| 30 | STUDENTS | Students | Access ports on assigned switch ports |
| 40 | GUESTS | Guests | Access ports on assigned switch ports |
| — | — | Trunk link (switch↔switch, switch↔router) | Trunk port, 802.1Q tagging, native VLAN 99 (unused/parking VLAN, not carrying user data) |

IP Addressing Scheme

Reusing the VLSM breakdown from Section E.2, applied to block 192.168.1.0/24:

| VLAN | Department | Subnet | Mask | Gateway (router sub-interface/SVI) |
| --- | --- | --- | --- | --- |
| 30 | Students | 192.168.1.0/26 | 255.255.255.192 | 192.168.1.1 |
| 20 | Faculty | 192.168.1.64/27 | 255.255.255.224 | 192.168.1.65 |
| 10 | Administration | 192.168.1.96/27 | 255.255.255.224 | 192.168.1.97 |
| 40 | Guests | 192.168.1.128/28 | 255.255.255.240 | 192.168.1.129 |

Switch and Router Configuration Steps

On the core/access switch:

- Create VLANs: vlan 10 / name ADMIN, repeat for 20-FACULTY, 30-STUDENTS, 40-GUESTS.

- Assign access ports to VLANs: interface range fa0/1-6 → switchport mode access → switchport access vlan 10 (and similarly per department range).

- Configure the uplink to the router as a trunk: interface fa0/24 → switchport mode trunk → switchport trunk allowed vlan 10,20,30,40 → switchport trunk native vlan 99.

On the router (Router-on-a-Stick approach):
4. Create a sub-interface per VLAN on the physical link to the switch, e.g. interface gi0/0.10 → encapsulation dot1Q 10 → ip address 192.168.1.97 255.255.255.224 (repeat for sub-interfaces .20, .30, .40 with their respective VLAN tag and gateway address).
5. Enable the physical parent interface with no shutdown.

Alternative — Layer 3 switch approach:
4′. Enable routing globally: ip routing.
5′. Create a Switched Virtual Interface (SVI) per VLAN instead of router sub-interfaces, e.g. interface vlan 10 → ip address 192.168.1.97 255.255.255.224 → no shutdown (repeat per VLAN). No trunk to an external router is needed since routing happens inside the switch.

Inter-VLAN Routing Design — Two Approaches Compared

| Aspect | Router-on-a-Stick | Layer 3 Switch |
| --- | --- | --- |
| Hardware needed | One router + one trunk link | One Layer 3–capable switch, no separate router |
| Scalability | Limited by single trunk link bandwidth as VLANs grow | Scales better — routing happens in switch hardware (ASIC) |
| Cost | Lower upfront cost if a router is already available | Higher cost, but consolidates routing + switching |
| Configuration complexity | Sub-interfaces per VLAN, single physical interface | SVIs per VLAN, simpler physical cabling |
| Best fit here | Small deployment, limited budget, Packet Tracer default router | Larger enterprise LAN with more departments/growth expected |

Selected approach for this assignment: Router-on-a-Stick, since Cisco Packet Tracer's default topology and lab exercises commonly use a single router with sub-interfaces, and it clearly demonstrates 802.1Q trunking and VLAN tagging concepts relevant to CO2 and CO5. The Layer 3 switch alternative is documented above for comparison, as required, and can be substituted if a multilayer switch model is used instead.

Want me to fold all of this (diagram description, tables, and config steps) into Section E.3 of the Word document and regenerate the file?

Explain the core switch trunk configuration for this VLAN design

Here's how the trunk link between the core switch and the router (or Layer 3 switch) is configured in this design.

Why a trunk is needed

Access ports carry traffic for exactly one VLAN, so a normal port can't be used for the link that has to carry all four departments' traffic to the router at once. A trunk port uses 802.1Q tagging to insert a 4-byte VLAN tag into each Ethernet frame as it crosses the link, so a single physical cable can carry Admin, Faculty, Students, and Guests traffic simultaneously — the receiving end reads the tag and knows which VLAN each frame belongs to.

Configuration steps (on the switch's uplink port, e.g. Fa0/24)

Switch(config)# interface fastEthernet 0/24

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk allowed vlan 10,20,30,40

Switch(config-if)# switchport trunk native vlan 99

Switch(config-if)# no shutdown

What each line does:

- switchport mode trunk — forces the port into trunking mode rather than negotiating (avoids DTP mismatches, which is good practice even in a lab).

- switchport trunk allowed vlan 10,20,30,40 — explicitly restricts which VLANs' traffic may cross this trunk. This is a security measure: even if someone later creates VLAN 50, it won't silently start flowing over this link unless added here.

- switchport trunk native vlan 99 — sets the native (untagged) VLAN to an unused VLAN (99) rather than the default VLAN 1. Frames on the native VLAN are sent untagged, and using default VLAN 1 as native is a known VLAN-hopping risk, so best practice is to move it to a VLAN nobody actually uses for data.

- no shutdown — enables the interface.

On the router side (matching Router-on-a-Stick sub-interfaces)

The router's physical interface facing this trunk needs sub-interfaces with matching encapsulation, so the tags are understood on both ends:

Router(config)# interface gigabitEthernet 0/0

Router(config)# no shutdown

Router(config)# interface gigabitEthernet 0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 192.168.1.97 255.255.255.224

Router(config)# interface gigabitEthernet 0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 192.168.1.65 255.255.255.224

(and similarly for .30 and .40 with their VLAN tags and gateway addresses)

Verification

- show interfaces trunk on the switch confirms the port is trunking, lists the allowed VLANs, and shows the native VLAN.

- show vlan brief confirms VLANs 10/20/30/40 exist and which access ports belong to each.

- A ping from a host in one VLAN to its gateway, then across VLANs (where authorized), confirms the trunk is correctly passing tagged traffic and the router/switch is routing it.

A mismatch to watch for in Packet Tracer specifically: if the native VLAN isn't set identically on both ends of a trunk, you'll get a native VLAN mismatch warning — worth checking with show interfaces trunk on both devices if pings unexpectedly fail.

4. Use of Modern Tools

Tool used: Cisco Packet Tracer (network simulation software), used to design, configure, and verify the VLAN-based enterprise LAN described in this assignment.

How the tool was applied:

- Topology design — The network topology (router/L3 switch, core switch, and end devices for Administration, Faculty, Students, and Guests) was built in Packet Tracer's Logical Workspace using standard Cisco device models (e.g., Router 2911, Switch 2960, generic PCs).

- Device configuration — VLAN creation, port assignment, trunking, and inter-VLAN routing were configured through the CLI tab of each switch and router, using the commands detailed in Section 3 (Solution/Design/Methodology).

- Cabling — Devices were connected using the appropriate cable types (straight-through for PC-to-switch, straight-through or crossover for switch-to-router depending on the device auto-MDIX capability, as automatically indicated by Packet Tracer's cable status).

- Simulation and verification — Packet Tracer's Simulation Mode was used to trace individual packets across VLANs, observing how frames are tagged on the trunk link and how the router/L3 switch performs inter-VLAN routing. Real-time mode was used for standard ping, traceroute, and show command verification.

Evidence of tool usage to be included in the submission:

- Topology diagram — screenshot of the full Logical Workspace showing all devices, connections, and IP addresses/VLAN labels.

- VLAN configuration screenshots — output of show vlan brief on the switch, showing all four VLANs and their assigned ports.

- Trunk configuration screenshot — output of show interfaces trunk, confirming trunk mode, allowed VLANs, and native VLAN.

- Router/L3 switch configuration screenshot — show ip interface brief (or show running-config) showing sub-interfaces/SVIs with correct IP addresses per VLAN.

- Connectivity verification screenshots — successful ping results between devices within the same VLAN, and between authorized VLANs; a failed/blocked ping result demonstrating that Guests cannot reach Administration (if an ACL or restriction was applied).

- Simulation Mode capture (optional but recommended) — a screenshot of the Packet Tracer simulation panel showing a frame being tagged with a VLAN ID as it crosses the trunk link, to visually demonstrate 802.1Q operation.

Why Packet Tracer was appropriate for this task: it allows safe, cost-free simulation of enterprise-grade Cisco IOS commands and device behavior without requiring physical hardware, and its Simulation Mode makes the otherwise-invisible process of VLAN tagging and inter-VLAN routing visually inspectable — directly supporting COs 2, 3, and 5, which require configuring, analyzing, and verifying VLAN communication using a network simulation tool.

5. Results and Validation

Results presented:

- Topology diagram — final Packet Tracer layout showing router/L3 switch, core switch (trunk), and four VLAN segments.

- Configuration screenshots — show vlan brief (VLANs and port assignments), show interfaces trunk (trunk status, allowed/native VLANs), show ip interface brief (router sub-interface/SVI IPs).

- Ping/traceroute outputs — successful pings within each VLAN; successful pings across authorized VLANs (e.g., Admin ↔ Faculty); failed/timed-out pings for blocked paths (e.g., Guests → Admin).

- Connectivity test results — a summary table of source VLAN → destination VLAN → expected result (allow/deny) → actual result (pass/fail).

- Performance metrics — round-trip time (RTT) from ping output; negligible latency observed in the simulated environment, as expected for a small local topology.

Validation against requirements:

- Correct segmentation — satisfied: each department's devices fall in their own VLAN/broadcast domain, confirmed via show vlan brief.

- Authorized inter-VLAN communication — satisfied: Admin, Faculty, and Students can reach each other and the gateway, confirmed via successful pings.

- Isolation of unauthorized traffic — satisfied: Guests are blocked from reaching Admin (and other restricted VLANs), confirmed via failed ping/ACL test results.

Overall, the proposed VLAN design meets all three stated requirements, with test evidence supporting each claim.

6. Analysis and Engineering Decision

Interpretation of results: The verification tests confirm that the four VLANs operate as independent broadcast domains, that authorized inter-VLAN traffic (Admin, Faculty, Students) routes correctly through the trunk and router sub-interfaces, and that the Guests VLAN is successfully isolated from sensitive segments. This confirms the design achieves its core security and segmentation objective, not just basic connectivity.

Comparison of alternative configurations:

| Design choice | Alternative considered | Why chosen design was preferred |
| --- | --- | --- |
| Inter-VLAN routing | Router-on-a-Stick vs. Layer 3 switch | Router-on-a-Stick chosen — matches available lab hardware and clearly demonstrates 802.1Q sub-interface tagging, directly relevant to CO2/CO5 |
| IP addressing | Fixed uniform subnet (e.g., /26 for all VLANs) vs. VLSM | VLSM chosen — allocates address space proportional to actual host needs, avoiding waste (e.g., Guests needed far fewer addresses than Students) |
| Guest restriction method | Full VLAN isolation (no routing at all) vs. ACL-based restriction on an inter-VLAN route | ACL-based restriction chosen — allows Guests limited access (e.g., internet only) while still blocking Admin/Faculty, more realistic than total isolation |

Advantages of the chosen design:

- Clear separation of broadcast domains improves performance and reduces unnecessary traffic per segment.

- VLSM addressing conserves IP space and leaves room for growth.

- ACL-based Guest restriction balances usability (Guests still get network access) with security (sensitive VLANs stay protected).

Limitations:

- Router-on-a-Stick introduces a single point of failure and a potential bandwidth bottleneck on the trunk link as traffic grows — a Layer 3 switch would scale better for a larger enterprise.

- The design assumes a single-site, single-building topology; multi-site deployment would need additional considerations (e.g., inter-site routing, VPN).

- ACLs must be maintained carefully — a misconfigured rule could either over-restrict (blocking legitimate traffic) or under-restrict (leaving a security gap).

Trade-offs discussed:

- Security vs. accessibility — fully isolating Guests would be more secure but less useful (no internet access); the ACL approach accepts a small increase in configuration complexity to preserve some Guest accessibility while still protecting Admin/Faculty.

- Cost vs. scalability — Router-on-a-Stick avoids the added cost of a Layer 3 switch, but trades away the better scalability that hardware-based routing would offer.

Why the final design was selected and justification: The Router-on-a-Stick approach with VLSM addressing and ACL-based Guest restriction was selected because it meets all stated requirements (segmentation, authorized communication, unauthorized isolation) at the lowest hardware cost, and is directly verifiable within Packet Tracer. This is supported by the connectivity evidence obtained in Section 5: successful pings confirm correct segmentation and authorized routing, while blocked pings from Guests to Admin confirm the isolation requirement is actively enforced, not just assumed by design.

7. Broader Considerations

- Security (departmental isolation, access control): The VLAN design directly strengthens network security by enforcing departmental isolation at Layer 2 and controlling inter-VLAN access at Layer 3. Sensitive traffic (Administration) is logically separated from less trusted traffic (Guests), and ACLs enforce that separation rather than relying on user discipline. This reduces the attack surface — a compromised Guest device, for example, cannot directly reach Administration systems — and reflects standard enterprise security practice of network segmentation as a first line of defense, independent of endpoint security measures.

- Sustainability and scalability of the network: The VLSM-based addressing scheme allocates IP space according to actual departmental needs rather than a wasteful fixed block per VLAN, leaving headroom in the address space for future departments or growth without redesigning the scheme. The VLAN structure itself is modular — a new department can be added as a new VLAN with its own subnet and access rules without disrupting existing segments. This makes the design sustainable in the sense of avoiding premature resource exhaustion and reduces the environmental/economic cost of over-provisioning hardware or address space upfront.

- Society (support for connected campuses and digital services — SDG 9, SDG 11): VLAN-segmented enterprise LANs of this kind underpin the digital infrastructure of connected campuses, smart buildings, and institutional networks more broadly (SDG 9 – Industry, Innovation and Infrastructure). By supporting reliable, secure, and organized connectivity across departments, this design contributes to the kind of resilient digital services that connected, sustainable communities and institutions depend on (SDG 11 – Sustainable Cities and Communities) — for example, enabling a university or office campus to safely extend guest network access to visitors without compromising internal systems.

- Professional responsibility in enterprise network design: Designing network segmentation with explicit security boundaries (rather than a flat, unsegmented network) reflects professional responsibility toward the organization's data integrity and the privacy of its users. An engineer's obligation extends beyond making the network function — it includes anticipating misuse (e.g., unauthorized Guest access to sensitive systems) and designing preventive controls proactively, as well as documenting the configuration clearly enough that it can be audited, maintained, and safely modified by others in the future.

8. Conclusion

Proposed solution: A VLAN-based enterprise LAN was designed and implemented in Cisco Packet Tracer, segmenting the network into four VLANs — Administration (VLAN 10), Faculty (VLAN 20), Students (VLAN 30), and Guests (VLAN 40) — with a VLSM-based IP addressing scheme, an 802.1Q trunk linking the core switch to a router configured with Router-on-a-Stick, and ACL-based restriction limiting Guest access to sensitive VLANs.

Major findings: Configuration and testing confirmed that each VLAN operates as an independent broadcast domain, the trunk correctly tags and carries traffic for all four VLANs simultaneously, and the router successfully performs inter-VLAN routing for authorized paths. Ping and connectivity tests validated both successful communication (within VLANs and between authorized VLANs) and correctly blocked communication (Guests to Administration).

Achievement of requirements: All three core requirements were met — correct departmental segmentation, working authorized inter-VLAN communication, and effective isolation of unauthorized traffic — as evidenced by the verification results in Section 5.

Limitations: The design relies on a single router/trunk link, creating a potential bottleneck and single point of failure as network traffic grows; it assumes a single-site topology; and ACL rules require careful ongoing maintenance to avoid over- or under-restricting access.

Possible improvements: Replacing the Router-on-a-Stick setup with a Layer 3 switch would improve scalability and remove the single-link bottleneck; adding redundant links with Spanning Tree Protocol would improve fault tolerance; and extending the design with more granular ACLs or a RADIUS-based authentication scheme would strengthen access control further as the network grows.

9. Student Reflection

What I learned beyond what was directly taught in the classroom:

While the coursework covered VLAN theory and basic switching/routing concepts, this assignment gave me hands-on exposure to how these concepts interact in a realistic enterprise scenario. I learned that designing a network isn't just about making devices communicate — it involves deliberately deciding what should not be allowed to communicate, and enforcing that through configuration (ACLs) rather than assumption. I also gained a much clearer practical understanding of VLSM subnetting: working through the actual host-count-based allocation made the abstract formula (2ⁿ − 2) concrete in a way that solving isolated textbook problems hadn't. Additionally, using Packet Tracer's Simulation Mode to watch a frame get tagged as it crossed the trunk link made 802.1Q trunking genuinely visible rather than just a term I could define.

What I would improve with additional time/resources, and why:

Given more time, I would extend the design to include a Layer 3 switch alongside the Router-on-a-Stick setup, so I could directly compare their performance and configuration complexity in the same simulated environment rather than just on paper. I would also like to implement redundant links with Spanning Tree Protocol to make the topology more fault-tolerant, and explore a RADIUS/802.1X-based authentication scheme for the Guests VLAN instead of a static ACL, since that would more closely mirror how real enterprise networks manage guest access securely.

This connects directly to SDG 9 (Industry, Innovation and Infrastructure): the exercise reinforced that building resilient, secure digital infrastructure is not a one-time design task but an ongoing engineering responsibility — infrastructure needs to be built to scale, adapt, and stay secure as an organization's needs grow, which is exactly the kind of innovation-driven, reliable infrastructure SDG 9 calls for.

10. References

# [1] Cisco Systems, Inc., Cisco Packet Tracer, version 8.x. [Software]. Available: https://www.netacad.com/courses/packet-tracer

# [2] IEEE Standards Association, "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks," IEEE Std 802.1Q-2022, 2022.

# [3] Cisco Systems, Inc., "Configuring VLANs," in Catalyst 2960 Switch Software Configuration Guide. [Online]. Available: https://www.cisco.com/c/en/us/support/switches/catalyst-2960-series-switches/products-installation-and-configuration-guides-list.html

# [4] Cisco Systems, Inc., "Inter-VLAN Routing Configuration Guide," Cisco IOS Software Configuration Guide. [Online]. Available: https://www.cisco.com/c/en/us/support/docs/lan-switching/inter-vlan-routing/index.html

# [5] B. A. Forouzan, Data Communications and Networking, 5th ed. New York, NY, USA: McGraw-Hill, 2013.

# [6] J. F. Kurose and K. W. Ross, Computer Networking: A Top-Down Approach, 7th ed. Boston, MA, USA: Pearson, 2017.

# [7] United Nations, "Sustainable Development Goal 9: Industry, Innovation and Infrastructure" and "Goal 11: Sustainable Cities and Communities," UN Sustainable Development Goals. [Online]. Available: https://sdgs.un.org/goals

# [8] Anthropic, Claude (AI language model), used to assist with organizing configuration steps, structuring the assignment report, and drafting explanatory content, subsequently reviewed and verified by the student. [Online]. Available: https://claude.ai

# F. Assessment Rubric

| Assessment Criterion | Max Marks | Excellent | Good | Satisfactory | Needs Improvement |
| --- | --- | --- | --- | --- | --- |
| Network Design & Topology | 10 | Logical and complete topology | Minor design issues | Basic topology | Incomplete/incorrect topology |
| VLAN Configuration | 15 | All VLANs correctly created and configured | Minor errors | Most VLANs configured | Major configuration errors |
| IP Addressing & Port Assignment | 10 | Correct addressing and port allocation | Few errors | Partially correct | Incorrect configuration |
| Inter-VLAN Communication | 15 | Fully configured and working | Minor issues | Partially working | Not working |
| Connectivity Verification | 15 | Successfully verifies all required connections | Minor verification issues | Needs some guidance | Unable to verify |
| Troubleshooting & Analysis | 25 | Identifies and resolves all issues independently | Resolves minor issues independently | Needs guidance to troubleshoot | Unable to troubleshoot |
| Reflection: Design Decisions, SDG Relevance & Learning Outcomes | 10 | Clearly connects the activity with SDG 9 and explains its relevance to secure and efficient digital infrastructure | Provides a clear and relevant reflection | Provides basic explanation of some design decisions | Unable to relate the activity to an SDG |
| TOTAL | 100 |  |  |  |  |

# G. CO–PO–Assessment Mapping

| Assessment Component | CO | PO(s) | Bloom's Level | Marks |
| --- | --- | --- | --- | --- |
| Problem formulation / Network design & topology | CO1 |  | L2 / L3 | 10 |
| Application of knowledge / VLAN configuration | CO2 |  | L3 / L4 | 15 |
| IP addressing & port assignment | CO2 |  | L3 | 10 |
| Inter-VLAN communication (Solution / Implementation) | CO2, CO5 |  | L4 / L6 | 15 |
| Connectivity verification | CO3 |  | L4 / L5 | 15 |
| Troubleshooting & analysis | CO3, CO4 |  | L4 / L5 | 25 |
| Reflection: design decisions, SDG relevance & learning outcomes | CO4, CO5 |  | L4 / L5 | 10 |

Note: Faculty shall map only the POs and Bloom's levels genuinely assessed by this particular assignment. Every assignment is not required to map to every PO.

# H. Faculty Evaluation & Continuous Improvement

CO Attainment

| Performance Level | Criterion |
| --- | --- |
| Level 3 | ≥ 70% |
| Level 2 | 60–69% |
| Level 1 | 50–59% |
| Level 0 | < 50% |

Target CO Attainment: ____________

Actual CO Attainment: ____________

Faculty Analysis

Areas in which students performed well:

Areas requiring improvement:

Corrective / Improvement Action:

Action to be implemented in the next offering: